Contingency Casing Design

If hole conditions dictate, 7" casing will be set at 7,173' MD (6,900' TVD). A 6 1/8" hole will then be drilled to 10,985' MD (6,900' TVD) where 4 1/2" casing will be set and cemented with one stage up to dv tool. After completion procedures, the 4 1/2" casing will be cut and pulled at 6300'

2nd Intermediate

|  | 0 ft to 100 ft | Make up Torque ft-lbs | Total ft = 100 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D | Weight | Grade | Threads | opt. | min. mx. |  |
| 7 inches | 26#/ft | J-55 | LT&C | 3670 | 2750 4590 |  |
| Collapse Resistance | Internal Yield | Joint Strength | Body Yield | Drift |  |  |
| 4,320 psi | 4,980 psi | 367,000# | 415,000# | 6.151 |  |  |

|  | 100 ft to 5,800 ft | Make up Torque ft-lbs | Total ft = 5,700 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. min | mx. |  |
| 7 inches | 23#/ft | J-55 | LT&C | 3130 | 2350 | 3910 |
| Collapse Resistance 3,270 | Internal Yield 4,360 psi | Joint Strength 313,000 # | Body Yield 366,000 # | Drift 6.25 |  |  |

|  | 5,800 ft to 7,173 ft | Make up Torque ft-lbs | Total ft = 1,373 |  |  |
| --- | --- | --- | --- | --- | --- |
| O.D | Weight | Grade | Threads | min. mx. |  |
| 7 inches | 26#/ft | J-55 | LT&C | 3670 2750 4590 |  |
| Collapse Resistance | Internal Yield | Joint Strength | Body Yield Drift |  |  |
| 4,320 psi | 4,980 psi | 367.000# | 415.000# 6.151 |  |  |

Lead w/750sx Lite crete (YLD 2.66 Wt. 9.9) tail w/125sx PVL (YLD 1.41 Wt 13) TOC = Surface

Production

|  | 0 ft to 10,985 ft | Make up Torque ft-lbs | Total ft = 10,985 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt | min. mx. |  |
| 4.5 inches | 11.6#/ft | HCP-110 | LT&C | 3020 | 2270 3780 |  |
| Collapse Resistance | Internal Yield | Joint Strength | Body Yield | Drift |  |  |
| 8,650 psi | 10,690 psi | 279,000# | 367,000# | 3.875 |  |  |

DV tool placed at approx. 6300' and cemented with one stage up to dv tool. After completion procedures, the 4 1/2" casing will be cut and pulled at 6300'.

Cemented w/625sx PVL (YLD 1.41 Wt 13) TOC= 6300'