12. Check Appropriate Box to Indicate Nature of Notice, Report or Other Data

| NOTICE OF INTENTION TO: | SUBSEQUENT REPORT OF: |  |  |  |
| --- | --- | --- | --- | --- |
| REFORM REMEDIAL WORK ☐ | PLUG AND ABANDON ☐ | REMEDIAL WORK ☐ | ALTERING CASING ☐ |  |
| MPORARILY ABANDON ☐ | CHANGE PLANS ☐ | COMMENCE DRILLING OPNS. ☐ | P AND A ☐ |  |
| ILL OR ALTER CASING ☐ | MULTIPLE COMPL ☐ | CASING/CEMENT JOB ☐ |  |  |
| DOWNHOLE COMMINGLE ☐ |  |  |  |  |
| THER: ☐ | OTHER: Surface and Intermediate casing ☒ |  |  |  |

OTHER:

13. Describe proposed or completed operations. (Clearly state all pertinent details, and give pertinent dates, including estimated date of starting any proposed work). SEE RULE 1103. For Multiple Completions: Attach wellbore diagram of proposed completion or recompletion.

4/30/10 – Resumed drilling at 11:20 PM.

$$ 17-1/2^{\prime \prime} $$

5/2/10 – Set 13-3/8" 48# H-40 ST&C casing at 805'. Cemented with 450 sx Class "C" + 0.125#/sx D130 + 3.76#/sx D20 + 94#/sx D903 + 1.88#/sx S1 + 0.188#/sx D46 (yld 1.97, wt 12.9). Tailed in with 200 sx Class "C" + 0.125#/sx D130 + 94#/sx D903 + 0.94#/sx S1 (yld 1.34, wt 14.8). Cement circulated to surface. Tested casing to 600 psi for 30 min, ending 540 psi. NOTE: NMOCD was notified via voicemail and no representative on location to witness cement job. WOC 20 hrs. Reduced hole to 12-1/4" and resumed drilling.

5/3/10 – TD 12-1/4" hole at 2:00 AM to 2286'. Set 9-5/8" 36# J-55 LT&C casing at 2286'. Notified Mike Bratcher-NMOCD of pending cement job but did not witness. Cemented with 550 sx 35:65:6 Poz "C" + 0.621#/sx D112 + 27.65#/sx D132 + 5.325#/sx D20 + 4.827#/sx D44 + 61.1#/sx D903 + 0.177#/sx D46 (yld 2.07, wt 12.6). Tailed in with 200 sx Class "C" + 94#/sx D903 + 0.1% D13 (yld 1.33, wt 14.8). Cement circulated to surface. Tested casing to 1200 psi for 30 min – no pressure loss. WOC 48 hrs. Reduced hole to 8-3/4" and resumed drilling.

Spud Date:

Rig Release Date:

I hereby certify that the information above is true and complete to the best of my knowledge and belief.

SIGNATURE

Regulatory Compliance Supervisor

For State Use Only

APPROVED BY: David May

Conditions of Approval (if any):

TITLE Field Supervisor

DATE 6-1-2010