Pathfinder

Pathfinder X & Y Survey Report

| Company: | Yates Petroleum Corp. | Local Co-ordinate Reference: | Well #1H |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Project: | Eddy County | TVD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |
| Site: | Perdomo "BMP" State | MD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |
| Well: | #1H | North Reference: | Grid |  |  |  |
| Wellbore: | OH | Survey Calculation Method: | Minimum Curvature |  |  |  |
| Design: | OH | Database: | Midland Database |  |  |  |
| Project | Eddy County |  |  |  |  |  |
| Map System: | US State Plane 1983 | System Datum: | Mean Sea Level |  |  |  |
| Geo Datum: | North American Datum 1983 |  |  |  |  |  |
| Map Zone: | New Mexico Eastern Zone |  |  |  |  |  |
| Site | Perdomo "BMP" State |  |  |  |  |  |
| Site Position: | Map | 0.00 ft | Northing: | 436.693.070 ft | Latitude: | 32° 12' 1.492 N |
| From: | Easting: | 597.818.111 ft | Longitude: | 104° 9' 2.637 W |  |  |
| Position Uncertainty: | Slot Radius: |  | Grid Convergence: | 0.10 ° |  |  |
| Well: | #1H |  |  |  |  |  |
| Well Position | +N/-S | 0.00 ft | Northing: | 436.693.070 ft | Latitude: | 32° 12' 1.492 N |
| +E/-W | 0.00 ft | Easting: | 597.818.111 ft | Longitude: | 104° 9' 2.637 W |  |
| Position Uncertainty | 0.00 ft | Wellhead Elevation: | ft |  | Ground Level: | 3,149.00 ft |
| Wellbore | OH |  |  |  |  |  |
| Magnetics | Model Name | Sample Date | Declination: | Dip Angle (°) | Field Strength (nT) |  |
|  | IGRF2010 | 04/19/2010 | 7.99 | 60.09 | 48,654 |  |
| Design | OH |  |  |  |  |  |
| Audit Notes: |  |  |  |  |  |  |
| Version: | 1.0 | Phase: | ACTUAL | Tie On Depth: | 0.00 |  |
| Vertical Section: | Depth From (TVD) (ft) | +N/-S (ft) | +E/-W (ft) | Direction (°) |  |  |
|  |  | 0.00 | 0.00 | 0.00 | 179.66 |  |
| Survey Program | Date | 05/20/2010 |  |  |  |  |
| From (ft) | To (ft) | Survey (Wellbore) | Tool Name | Description |  |  |
| 200.00 | 8,600.00 | Pilot Hole (OH) | MWD | MWD - Standard |  |  |