| Company: | Yates Petroleum Corp. | Local Co-ordinate Reference: | Well #1H |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project: | Eddy County | TVD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |  |  |  |  |  |
| Site: | Perdomo "BMP" State | MD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |  |  |  |  |  |
| Well: | #1H | North Reference: | Grid |  |  |  |  |  |  |  |  |
| Wellbore: | OH | Survey Calculation Method: | Minimum Curvature |  |  |  |  |  |  |  |  |
| Design: | OH | Database: | Midland Database |  |  |  |  |  |  |  |  |
| Survey |  |  |  |  |  |  |  |  |  |  |  |
| MD | Inc | Azi | TVD | TVDSS | N/S | E/W: | V: Sec | DLeg | Northing | Easting | Th |
| (ft) | (°) | (°) | (ft) | (ft) | (ft) | (ft) | (ft) | (°/100ft) | (ft) | (ft) |  |
| 4,232.00 | 0.53 | 245.01 | 4,231.16 | 1,055.66 | 19.93 | 2.00 | -19.92 | 0.10 | 436,713.00 | 597,820.11 |  |
| 4,421.00 | 1.14 | 174.96 | 4,420.14 | 1,244.64 | 17.69 | 1.37 | -17.68 | 0.57 | 436,710.76 | 597,819.48 |  |
| 4,608.00 | 1.85 | 192.45 | 4,607.08 | 1,431.58 | 12.89 | 0.88 | -12.89 | 0.45 | 436,705.96 | 597,818.99 |  |
| 4,796.00 | 0.35 | 215.21 | 4,795.04 | 1,619.54 | 9.46 | -0.10 | -9.46 | 0.82 | 436,702.53 | 597,818.01 |  |
| 4,984.00 | 0.44 | 224.18 | 4,983.03 | 1,807.53 | 8.47 | -0.94 | -8.48 | 0.06 | 436,701.54 | 597,817.17 |  |
| 5,173.00 | 0.62 | 234.73 | 5,172.02 | 1,996.52 | 7.36 | -2.28 | -7.37 | 0.11 | 436,700.43 | 597,815.83 |  |
| 5,360.00 | 0.79 | 231.83 | 5,359.01 | 2,183.51 | 5.98 | -4.12 | -6.00 | 0.09 | 436,699.05 | 597,813.99 |  |
| 5,548.00 | 0.79 | 221.63 | 5,546.99 | 2,371.49 | 4.21 | -6.00 | -4.25 | 0.07 | 436,697.28 | 597,812.11 |  |
| 5,736.00 | 0.97 | 207.83 | 5,734.97 | 2,559.47 | 1.83 | -7.60 | -1.88 | 0.15 | 436,694.90 | 597,810.51 |  |
| 5,766.00 | 1.67 | 185.60 | 5,764.96 | 2,589.46 | 1.18 | -7.76 | -1.22 | 2.85 | 436,694.25 | 597,810.35 |  |
| 5,798.00 | 3.43 | 172.32 | 5,796.93 | 2,621.43 | -0.24 | -7.68 | 0.19 | 5.77 | 436,692.83 | 597,810.43 |  |
| 5,829.00 | 7.56 | 168.10 | 5,827.78 | 2,652.28 | -3.15 | -7.14 | 3.11 | 13.38 | 436,689.92 | 597,810.98 |  |
| 5,860.00 | 12.93 | 168.72 | 5,858.27 | 2,682.77 | -8.55 | -6.04 | 8.52 | 17.33 | 436,684.52 | 597,812.07 |  |
| 5,892.00 | 18.91 | 169.60 | 5,889.03 | 2,713.53 | -17.17 | -4.40 | 17.15 | 18.70 | 436,675.90 | 597,813.71 |  |
| 5,924.00 | 22.77 | 170.92 | 5,918.93 | 2,743.43 | -28.39 | -2.48 | 28.38 | 12.15 | 436,664.68 | 597,815.63 |  |
| 5,955.00 | 25.68 | 171.71 | 5,947.20 | 2,771.70 | -40.97 | -0.57 | 40.96 | 9.44 | 436,652.10 | 597,817.54 |  |
| 5,986.00 | 28.05 | 174.17 | 5,974.85 | 2,799.35 | -54.87 | 1.14 | 54.87 | 8.44 | 436,638.20 | 597,819.25 |  |
| 6,017.00 | 31.66 | 177.24 | 6,001.74 | 2,826.24 | -70.25 | 2.27 | 70.26 | 12.64 | 436,622.82 | 597,820.38 |  |
| 6,049.00 | 35.26 | 178.92 | 6,028.43 | 2,852.93 | -87.88 | 2.85 | 87.89 | 11.62 | 436,605.19 | 597,820.96 |  |
| 6,080.00 | 38.60 | 179.62 | 6,053.21 | 2,877.71 | -106.50 | 3.08 | 106.52 | 10.86 | 436,586.57 | 597,821.20 |  |
| 6,111.00 | 41.86 | 180.06 | 6,076.87 | 2,901.37 | -126.52 | 3.14 | 126.54 | 10.56 | 436,566.55 | 597,821.25 |  |
| 6,143.00 | 45.64 | 180.23 | 6,099.98 | 2,924.48 | -148.64 | 3.08 | 148.66 | 11.82 | 436,544.43 | 597,821.19 |  |
| 6,174.00 | 49.77 | 179.97 | 6,120.84 | 2,945.34 | -171.57 | 3.04 | 171.58 | 13.34 | 436,521.50 | 597,821.15 |  |
| 6,205.00 | 54.34 | 179.71 | 6,139.90 | 2,964.40 | -196.01 | 3.11 | 196.02 | 14.76 | 436,497.06 | 597,821.22 |  |
| 6,237.00 | 59.09 | 179.62 | 6,157.45 | 2,981.95 | -222.75 | 3.27 | 222.77 | 14.85 | 436,470.32 | 597,821.38 |  |
| 6,268.00 | 63.31 | 180.59 | 6,172.38 | 2,996.88 | -249.91 | 3.21 | 249.92 | 13.89 | 436,443.16 | 597,821.33 |  |
| 6,299.00 | 67.18 | 181.11 | 6,185.36 | 3,009.86 | -278.05 | 2.80 | 278.06 | 12.58 | 436,415.02 | 597,820.91 |  |