| Company: | Yates Petroleum Corp. | Local Co-ordinate Reference: | Well #1H |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project: | Eddy County | TVD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |  |  |  |  |  |
| Site: | Perdomo "BMP" State | MD Reference: | WELL @ 3175.50ft (Original Well Elev) |  |  |  |  |  |  |  |  |
| Well: | #1H | North Reference: | Grid |  |  |  |  |  |  |  |  |
| Wellbore: | OH | Survey Calculation Method: | Minimum Curvature |  |  |  |  |  |  |  |  |
| Design: | OH | Database: | Midland Database |  |  |  |  |  |  |  |  |
| Survey |  |  |  |  |  |  |  |  |  |  |  |
| MD | Inc (f) | Azi (f) | TVD (ft) | TVDSS (ft) | N/S (ft) | E/W (ft) | V. Sec (ft) | DLeg (*/100ft) | Northing (ft) | Easting (ft) |  |
| 8,705.00 | 89.52 | 178.83 | 6,216.89 | 3,041.39 | -2,679.02 | -1.98 | 2,678.96 | 2.01 | 434,014.05 | 597,816.13 |  |
| 8,799.00 | 89.16 | 178.83 | 6,217.97 | 3,042.47 | -2,772.99 | -0.06 | 2,772.94 | 0.38 | 433,920.08 | 597,818.05 |  |
| 8,892.00 | 89.69 | 177.86 | 6,218.91 | 3,043.41 | -2,865.95 | 2.62 | 2,865.91 | 1.19 | 433,827.12 | 597,820.73 |  |
| 8,986.00 | 91.01 | 178.21 | 6,218.33 | 3,042.83 | -2,959.89 | 5.84 | 2,959.87 | 1.45 | 433,733.18 | 597,823.96 |  |
| 9,081.00 | 91.01 | 177.86 | 6,216.66 | 3,041.16 | -3,054.82 | 9.10 | 3,054.82 | 0.37 | 433,638.25 | 597,827.21 |  |
| 9,175.00 | 91.19 | 178.21 | 6,214.85 | 3,039.35 | -3,148.74 | 12.32 | 3,148.76 | 0.42 | 433,544.33 | 597,830.44 |  |
| 9,269.00 | 90.40 | 179.09 | 6,213.55 | 3,038.05 | -3,242.71 | 14.54 | 3,242.73 | 1.26 | 433,450.36 | 597,832.65 |  |
| 9,363.00 | 90.31 | 178.74 | 6,212.97 | 3,037.47 | -3,336.69 | 16.32 | 3,336.72 | 0.38 | 433,356.38 | 597,834.43 |  |
| 9,456.00 | 89.78 | 178.92 | 6,212.89 | 3,037.39 | -3,429.67 | 18.22 | 3,429.71 | 0.60 | 433,263.40 | 597,836.33 |  |
| 9,550.00 | 89.34 | 179.62 | 6,213.62 | 3,038.12 | -3,523.66 | 19.41 | 3,523.71 | 0.88 | 433,169.41 | 597,837.53 |  |
| 9,644.00 | 89.08 | 179.97 | 6,214.91 | 3,039.41 | -3,617.65 | 19.75 | 3,617.70 | 0.46 | 433,075.42 | 597,837.86 |  |
| 9,738.00 | 91.01 | 180.32 | 6,214.84 | 3,039.34 | -3,711.64 | 19.51 | 3,711.69 | 2.09 | 432,981.43 | 597,837.62 |  |
| 9,832.00 | 90.75 | 180.32 | 6,213.39 | 3,037.89 | -3,805.63 | 18.99 | 3,805.67 | 0.28 | 432,887.44 | 597,837.10 |  |
| 9,927.00 | 90.13 | 179.97 | 6,212.67 | 3,037.17 | -3,900.62 | 18.75 | 3,900.67 | 0.75 | 432,792.45 | 597,836.86 |  |
| 10,021.00 | 90.75 | 180.67 | 6,211.94 | 3,036.44 | -3,994.62 | 18.22 | 3,994.66 | 0.99 | 432,698.45 | 597,836.33 |  |
| 10,116.00 | 89.34 | 179.71 | 6,211.87 | 3,036.37 | -4,089.62 | 17.91 | 4,089.65 | 1.80 | 432,603.45 | 597,836.02 |  |
| 10,209.00 | 88.90 | 179.79 | 6,213.30 | 3,037.80 | -4,182.60 | 18.31 | 4,182.64 | 0.48 | 432,510.47 | 597,836.42 |  |
| 10,303.00 | 88.64 | 179.79 | 6,215.31 | 3,039.81 | -4,276.58 | 18.66 | 4,276.62 | 0.28 | 432,416.49 | 597,836.77 |  |
| 10,397.00 | 88.81 | 180.50 | 6,217.41 | 3,041.91 | -4,370.56 | 18.42 | 4,370.59 | 0.78 | 432,322.51 | 597,836.53 |  |
| 10,491.00 | 88.90 | 180.50 | 6,219.28 | 3,043.78 | -4,464.53 | 17.60 | 4,464.56 | 0.10 | 432,228.54 | 597,835.71 |  |
| 10,586.00 | 88.02 | 180.85 | 6,221.84 | 3,046.34 | -4,559.49 | 16.48 | 4,559.51 | 1.00 | 432,133.58 | 597,834.59 |  |
| 10,680.00 | 89.16 | 180.41 | 6,224.15 | 3,048.65 | -4,653.46 | 15.45 | 4,653.47 | 1.30 | 432,039.61 | 597,833.56 |  |
| 10,774.00 | 90.57 | 180.94 | 6,224.37 | 3,048.87 | -4,747.45 | 14.34 | 4,747.45 | 1.60 | 431,945.62 | 597,832.45 |  |
| 10,868.00 | 92.33 | 179.27 | 6,221.99 | 3,046.49 | -4,841.41 | 14.17 | 4,841.41 | 2.58 | 431,851.66 | 597,832.28 |  |
| 10,962.00 | 92.15 | 179.62 | 6,218.32 | 3,042.82 | -4,935.33 | 15.08 | 4,935.34 | 0.42 | 431,757.74 | 597,833.19 |  |
| 11,056.00 | 92.33 | 179.62 | 6,214.65 | 3,039.15 | -5,029.26 | 15.70 | 5,029.26 | 0.19 | 431,663.81 | 597,833.81 |  |
| 11,150.00 | 91.80 | 179.20 | 6,211.26 | 3,035.76 | -5,123.19 | 16.67 | 5,123.20 | 0.72 | 431,569.88 | 597,834.78 |  |