PathFinder Energy Services, Inc.

Survey Report

Page 01/02

Tie-in Date: 05/03/2010

YATES PETROLEUM

PERDOMO BMP STATE COM #1H - PILOT HOLE

EDDY COUNTY, NM

Rig:PATRIOT #5

PathFinder Office Supervisor: FRANCISCO SINISTERRA

PathFinder Field Engineers: Derek Boyd

James Cloud

Date Completed: 05/10/2010

Ref Coordinates: LAT:32.12.1.4925 N LON:104.9.2.6375 W

Survey Horiz. Reference: WELLHEAD

GRID Reference:NAD83 new mexico east Transverse Mercator

| Measured Depth (ft) | Incl (deg) | Drift Dir. (deg) | TVD (ft) | Course Length (ft) | Vertical Section (ft) | TOTAL Rectangular Offsets (ft) | Closure Dist (ft) | Dis Dist (deg) | DLS (dg/100ft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TIE-IN TO GYROSCOPIC DIRECTIONAL SURVEY @ 800' MD. |  |  |  |  |  |  |  |  |  |
| 800.00 | 0.75 | 340.52 | 799.98 | 0.00 | -2.36 | 2.35 N | 1.38 W | 2.73@ | 329.58 |
| THE FOLLOWING ARE PATHFINDER ENERGY SERVICES MWD SURVEYS: |  |  |  |  |  |  |  |  |  |
| 853.00 | 0.97 | 352.84 | 852.97 | 53.00 | -3.13 | 3.12 N | 1.55 W | 3.49@ | 333.57 |
| 1041.00 | 0.09 | 122.84 | 1040.97 | 188.00 | -4.63 | 4.62 N | 1.63 W | 4.90@ | 340.62 |
| 1230.00 | 0.62 | 240.79 | 1229.96 | 189.00 | -4.06 | 4.04 N | 2.39 W | 4.70@ | 329.36 |
| 1419.00 | 0.35 | 284.82 | 1418.96 | 189.00 | -3.71 | 3.69 N | 3.84 W | 5.33@ | 313.83 |
| 1604.00 | 0.35 | 104.30 | 1603.95 | 185.00 | -3.72 | 3.70 N | 3.84 W | 5.33@ | 313.88 |
| 1701.00 | 2.64 | 64.66 | 1700.92 | 97.00 | -4.59 | 4.58 N | 1.54 W | 4.83@ | 341.45 |
| 1795.00 | 3.61 | 65.10 | 1794.78 | 94.00 | -6.73 | 6.75 N | 3.10 E | 7.43@ | 24.70 |
| 1888.00 | 2.90 | 35.21 | 1887.63 | 93.00 | -9.86 | 9.91 N | 7.12 E | 12.20@ | 35.70 |
| 1982.00 | 3.43 | 55.87 | 1981.49 | 94.00 | -13.36 | 13.43 N | 10.82 E | 17.24@ | 38.85 |
| 2076.00 | 2.11 | 112.82 | 2075.39 | 94.00 | -14.25 | 14.33 N | 14.74 E | 20.56@ | 45.80 |
| 2170.00 | 1.85 | 178.21 | 2169.35 | 94.00 | -12.05 | 12.15 N | 16.38 E | 20.39@ | 53.45 |
| 2212.00 | 0.88 | 183.22 | 2211.34 | 42.00 | -11.05 | 11.15 N | 16.38 E | 19.82@ | 55.77 |
| 2350.00 | 0.88 | 30.12 | 2349.33 | 138.00 | -10.90 | 11.00 N | 16.86 E | 20.13@ | 56.86 |
| 2538.00 | 0.70 | 11.22 | 2537.31 | 188.00 | -13.27 | 13.38 N | 17.80 E | 22.27@ | 53.08 |
| 2726.00 | 1.06 | 7.09 | 2725.29 | 188.00 | -16.12 | 16.23 N | 18.24 E | 24.42@ | 48.34 |
| 2914.00 | 0.70 | 332.90 | 2913.27 | 188.00 | -18.87 | 18.98 N | 17.93 E | 26.11@ | 43.38 |
| 3103.00 | 0.70 | 308.82 | 3102.26 | 189.00 | -20.63 | 20.73 N | 16.51 E | 26.50@ | 38.53 |
| 3291.00 | 0.79 | 279.20 | 3290.24 | 188.00 | -21.57 | 21.66 N | 14.33 E | 25.97@ | 33.50 |
| 3479.00 | 0.79 | 271.29 | 3478.22 | 188.00 | -21.82 | 21.89 N | 11.76 E | 24.85@ | 28.24 |
| 3667.00 | 0.88 | 269.09 | 3666.20 | 188.00 | -21.85 | 21.90 N | 9.02 E | 23.69@ | 22.39 |
| 3855.00 | 0.88 | 263.99 | 3854.18 | 188.00 | -21.69 | 21.73 N | 6.14 E | 22.58@ | 15.78 |
| 4044.00 | 0.70 | 236.31 | 4043.16 | 189.00 | -20.91 | 20.93 N | 3.74 E | 21.27@ | 10.12 |
| 4232.00 | 0.53 | 245.01 | 4231.15 | 188.00 | -19.92 | 19.93 N | 1.99 E | 20.03@ | 5.71 |