| Submit To Appropriate District Office Two Copies District I 1625 N. French Dr., Hobbs, NM 88240 District II 1301 W. Grand Avenue, Anesia, NM 88210 District III 1000 Rio Brazos Rd., Aztee, NM 87410 District IV 1220 S. St. Francis Dr., Santa Fe, NM 87505 | RECEIVE State of New Mexico Energ. Minerals and Natural Resources | Form C-105 July 17, 2008 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEP 29, 2020 Oil Conservation Division HOBBSOO 020 South St. Francis Dr. Santa Fe, NM 87505 | 1. WELL API NO. 30-015-36499 |  |  |  |  |  |  |  |  |
| 2. Type of Lease ☒ STATE ☐ FEE ☐ FED/INDIAN |  |  |  |  |  |  |  |  |  |
| 3. State Oil & Gas Lease No. VB-827 & VO-5867 |  |  |  |  |  |  |  |  |  |
| WELL COMPLETION OR RECOMPLETION REPORT AND LOG |  |  |  |  |  |  |  |  |  |
| 4. Reason for filing: ☒ COMPLETION REPORT (Fill in boxes #1 through #31 for State and Fee wells only) ☐ C-144 CLOSURE ATTACHMENT (Fill in boxes #1 through #9, #15 Date Rig Released and #32 and/or #33; attach this and the plat to the C-144 closure report in accordance with 19.15.17.13.K NMAC) | 5. Lease Name or Unit Agreement Name Perdomo BMP State Com 6. Well Number: 1H |  |  |  |  |  |  |  |  |
| 7. Type of Completion: ☒ NEW WELL ☐ WORKOVER ☐ DEEPENING ☐ PLUGBACK ☐ DIFFERENT RESERVOIR ☐ OTHER |  |  |  |  |  |  |  |  |  |
| 8. Name of Operator Yates Petroleum Corporation | 9. OGRID 025575 |  |  |  |  |  |  |  |  |
| 10. Address of Operator 105 South Fourth Street, Artesia, NM 88210 | 11. Pool name or Wildcat WillowCafe, West Widgett, Bone Spring, off |  |  |  |  |  |  |  |  |
| 12. Location | Unit Lir | Section | Township | Range | Lot | Feet from the | N/S Line | Feet from the | E/W Line |
| Surface: | L | 24 | 24S | 27E |  | 1650 | South | 660 | West |
| BH: | M | 25 | 24S | 27E |  | 5125 | South | 667 | West |
| 13. Date Spudded RH 8/1/08 RT 4/30/10 | 14. Date T.D. Reached 5/20/10 | 15. Date Rig Released 5/23/10 | 16. Date Completed (Ready to Produce) 8/21/10 | 17. Elevations (DF and RKB, RT, GR, etc.) 3149'GR 3175'KB |  |  |  |  |  |
| 18. Total Measured Depth of Well 12,797' | 19. Plug Back Measured Depth 12,749' | 20. Was Directional Survey Made? Yes (Attached) | 21. Type Electric and Other Logs Run CNL, Hi-Res Laterolog Array, CBL |  |  |  |  |  |  |
| 22. Producing Interval(s), of this completion - Top, Bottom, Name 6437-12,747' Bone Spring |  |  |  |  |  |  |  |  |  |
| 23. | CASING RECORD (Report all strings set in well) |  |  |  |  |  |  |  |  |
| CASING SIZE | WEIGHT LB./FT. | DEPTH SET | HOLE SIZE | CEMENTING RECORD | AMOUNT PULLED |  |  |  |  |
| 20" | Conductor | 90' | 26" | Redi-mix |  |  |  |  |  |
| 13-3/8" | 48# | 805' | 17-1/2" | 650 sx (circ) |  |  |  |  |  |
| 9-5/8" | 36# | 2286' | 12-1/4" | 750 sx (circ) |  |  |  |  |  |
| 5-1/2" | 17# | 12,797' | 8-3/4" | 2010 sx |  |  |  |  |  |
| 24. | LINER RECORD | 25. TUBING RECORD |  |  |  |  |  |  |  |
| SIZE | TOP | BOTTOM | SACKS CEMENT | SCREEN | SIZE | DEPTH SET | PACKER SET |  |  |
|  |  |  |  |  |  | 2-7/8" | 5645' |  |  |
| 26. Perforation record (interval, size, and number) | 27. ACID, SHOT, FRACTURE, CEMENT, SOUEEZE, ETC. |  |  |  |  |  |  |  |  |
| SEE ATTACHED SHEET | DEPTH INTERVAL | AMOUNT AND KIND MATERIAL USED |  |  |  |  |  |  |  |
| 28. | PRODUCTION |  |  |  |  |  |  |  |  |
| Date First Production 8/22/10 | Production Method (Flowing, gas lift, pumping - Size and type pump) | Well Status (Prod. or Shut-in) Producing |  |  |  |  |  |  |  |
| Date of Test 8/24/10 | Hours Tested 24 | Choke Size 1" | Prod'n For Test Period | Oil - Bbl 138 | Gas - MCF 2815 | Water - Bbl. 2058 | Gas - Oil Ratio NA |  |  |
| Flow Tubing Press. 660 psi | Casing Pressure Packer | Calculated 24- Hour Rate | Oil - Bbl. 138 | Gas - MCF 2815 | Water - Bbl. 2058 | Oil Gravity - API - (Corr.) NA |  |  |  |
| 29. Disposition of Gas (Sold, used for fuel, vented, etc.) | 30. Test Winessed By R. Smith |  |  |  |  |  |  |  |  |
| 31. List Attachments |  |  |  |  |  |  |  |  |  |
| Logs, Deviation and Directional Surveys |  |  |  |  |  |  |  |  |  |
| 32. If an on-site burial was used at the well, attach a plat with the location of the temporary pit. |  |  |  |  |  |  |  |  |  |
| 33. If an on-site burial was used at the well, report the exact location of the on-site burial: Latitude Longitude NAD 1927.1983 |  |  |  |  |  |  |  |  |  |
| I hereby certify that the information shown on both sides of this form is true and complete to the best of my knowledge and belief. Signature Name Title Regulatory Compliance Supervisor Date August 26, 2010 |  |  |  |  |  |  |  |  |  |
| E-mail Address tinh@vatespetroleum.com |  |  |  |  |  |  |  |  |  |