Form C-103

12. Check Appropriate Box to Indicate Nature of Notice, Report or Other Data

| NOTICE OF INTENTION TO: | SUBSEQUENT REPORT OF: |  |  |  |
| --- | --- | --- | --- | --- |
| PERFORM REMEDIAL WORK ☐ | PLUG AND ABANDON ☐ | REMEDIAL WORK ☐ | ALTERING CASING ☐ |  |
| TEMPORARILY ABANDON ☐ | CHANGE PLANS ☐ | COMMENCE DRILLING OPNS. ☐ | P AND A ☐ |  |
| PULL OR ALTER CASING ☐ | MULTIPLE COMPL ☐ | CASING/CEMENT JOB ☐ |  |  |
| DOWNHOLE COMMINGLE ☐ |  |  |  |  |
| OTHER: | ☐ | OTHER: Production casing ☒ |  |  |

13. Describe proposed or completed operations. (Clearly state all pertinent details, and give pertinent dates, including estimated date of starting any proposed work). SEE RULE 19.15.7.14 NMAC. For Multiple Completions: Attach wellbore diagram of proposed completion or recompletion.

6/28/10 – TD pilot hole to 8500' at 6:00 PM.

6/30/10 – Set isolation plug from 6600'-6815' with 120 sx Class "H" with additives. Set kick off plug from 5380'-5880' with 300 sx Class "H" with additives. WOC. Tagged cement at 5569', slide to 5710' and started directionally drilling.

7/13/10 – Set 5-1/2” 17# L-80 LT&C 8rd casing at 10,490'. Float collar at 10,444'. Cemented stage 1 with 700 sx PVL + 0.2% D112 + 30% D151 + 2% D174 + 0.01 GPS D177 + 0.2% D46 + 0.8% D800 + 0.5% D079 (yld 1.83, wt 13). Circulated 107 sx cement off DV tool. Cemented stage 2 with 680 sx 35:65:6 Poz “C” + 0.2% D13 + 0.125#/sx D130 + 0.01 GPS D177 + 6% D20 + 2#/sx D42 + 5% D44 + 0.2% D46 (yld 2.07, wt 12.6).

Rig Release Date:

I hereby certify that the information above is true and complete to the best of my knowledge and belief.

Conditions of Approval (if any):