Page 01/01, Tie-in Date: 06/19/2010

Date Completed: 06/21/2010

JERICHO BKJ STATE COM #2H - STRAIGHT HOLE CONTROL

EDDY COUNTY, NEW MEXICO

Survey Horiz. Reference:WELLHEAD

Ref Coordinates: LAT:32.8.7.9146 N LON:104.10.14.5000 W

GRID Reference: NAD83 new mexico east Transverse Mercator

Ref GRID Coor: X: 591679.3420 Y: 413080.2210

GABE GARCIA

Total Magnetic Correction: 7.88° EAST TO

Survey Calculations by PathCalc v2.00 using Minimum Curvature

Vertical Section Plane: U.00

Survey Vert. Reference: 18.00' Kelly Bushing To Ground

Altitude: 3172.00' Ground To MSL

| Measured Depth (ft) | Incl (deg) | Drift Dir. (deg) | TVD (ft) | Course Length (ft) | Vertical Section (ft) | TOTAL Rectangular Offsets (ft) | Closure Dist (ft) | DIs (deg) | DLS (dg/100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASSUMING VERTICAL DEPTH @ 422' MD. |  |  |  |  |  |  |  |  |  |
| 422.00 | 0.00 | 0.00 | 422.00 | 0.00 | 0.00 | 0.00 | 0.00@ | 0.00 | 0.00 |
| THE FOLLOWING ARE PATHFINDER MWD SURVEYS. |  |  |  |  |  |  |  |  |  |
| 522.00 | 1.23 | 279.05 | 521.99 | 100.00 | 0.17 | 0.17 N | 1.06 W | 1.07@ | 279.05 |
| 706.00 | 2.29 | 283.15 | 705.90 | 184.00 | 1.32 | 1.32 N | 6.59 W | 6.72@ | 281.29 |
| 890.00 | 1.23 | 282.19 | 889.81 | 184.00 | 2.57 | 2.57 N | 12.10 W | 12.37@ | 281.99 |
| 1074.00 | 1.49 | 281.00 | 1073.76 | 184.00 | 3.44 | 3.44 N | 16.38 W | 16.74@ | 281.87 |
| 1257.00 | 2.11 | 280.64 | 1256.67 | 183.00 | 4.52 | 4.52 N | 22.03 W | 22.48@ | 281.59 |
| 1449.00 | 2.55 | 272.64 | 1448.51 | 192.00 | 5.37 | 5.37 N | 29.77 W | 30.25@ | 280.22 |
| 1639.00 | 0.62 | 220.50 | 1638.44 | 190.00 | 4.78 | 4.78 N | 34.66 W | 34.98@ | 277.85 |
| 1830.00 | 1.41 | 119.44 | 1829.41 | 191.00 | 2.84 | 2.84 N | 33.28 W | 33.40@ | 274.88 |
| 2019.00 | 3.34 | 103.73 | 2018.24 | 189.00 | 0.39 | 0.39 N | 25.91 W | 25.91@ | 270.86 |
| 2178.00 | 2.46 | 82.33 | 2177.04 | 159.00 | -0.25 | 0.25 S | 18.03 W | 18.03@ | 269.19 |
| STRAIGHT LINE PROJECTION TO THE BIT @ 2232' MD. |  |  |  |  |  |  |  |  |  |
| 2232.00 | 2.46 | 82.33 | 2230.99 | 54.00 | 0.06 | 0.06 N | 15.73 W | 15.73@ | 270.20 |