eog resources

Jericho BJK State Com #2H – P&A Procedure

Sec. 15, T. 25S, R.27E

API # 30-015-37500

APPROVED GC

5/20/26

1. Notify Regulatory Agency 24 hours prior to commencing work. MIRU well service unit and all necessary safety equipment.

2. ND WH, NU BOP.

3. POOH 2-3/8 4.7# L80 tbg. Stand back as plugging string.

4. Set 5.5" 17# CIBP @ 5710 (KOP), TAG, Pressure test casing at 500psi for 30 minutes. Bubble Test

5. PLUG: Spot 35 sxs class C on CIBP from 5710 5356. WOC/TAG. Covers CIBP and DV tool.

6. Circulate plugging mud

7. POOH and run RCBL with zero psi, keep hole loaded, log to surface

8. PLUG: Spot 25 sxs class C from 4300 – 4047. This covers Brushy Canyon.

9. PLUG: Spot 25 sxs class C from 3224 – 2971. This covers Cherry Canyon.

10. Perf at 2282. Attempt to inject.

11. PLUG: If able to inject, sqz 102 sxs class C from 2408'-2040. 24 sxs in, 78 sxs out. WOC/TAG. This covers int. casing shoe, B Salt and top of Delaware & Bell Canyon.

12. Perf at 784. Attempt to inject.

13. PLUG: Squeeze 64 sxs from 784' to 632'. 15 sxs in, 49 sxs out. This covers top of salt.

14. Perf at 520. Attempt to inject.

15. PLUG: Sqz 51 sxs from 520' to 399'. 12 sxs in, 39 sxs out. WOC/TAG.

16. Perf at 150. Attempt to circulate.

17. PLUG: Sqz 64 sxs from 150' to 0'. 15 sxs inside, 49 sxs outside. Surface to surface. WOC & Bubble Test

18. Cut off WH 3' below surface, verify cement to surface, and weld on below ground P&A marker.

19. Cut off anchors 3' below surface and clean location.