Well Name: Jericho BKJ State Com #2H

County: Eddy, NM

og resources

Current Wellbore Diagram:

KB: 3190

GL: 3172

17-1/2" Hole

13-3/8" 48# H-40 @ 470'

Cmt w/ 365 sx Class C (circ) to surface

12-1/4" Hole

9-5/8" 36# J-55 to 2232'

Cmt w/ 760 sx Class C (circ) to surface

KOP @ 5710

DV tool @ 5,496'

Pilot hole plug from 5380' - 5880'

300 sxs class H

Tag at 5569, slide to 5710, then start directional drill

8-3/4" Pilot Hole to 8497' TVD

Cement Plug @ 6600'-6815', 120 sxs class H

| Formation Tops |  |
| --- | --- |
| Top of Salt | 734 |
| Bottom of Salt | 2117 |
| Bell Canyon | 2358 |
| Cherry Canyon | 3174 |
| Brushy Canyon | 4250 |
| Bone Spring | 5828 |
| Leonard | 5926 |

Bottom Perf @ 10,434'

TD 10495

Pilot Hole TD @ 8.500'

8-3/4" Lateral Hole

5.5" 17# L80 Prod CSG

to 10,490'. 1380 sx cmt

PBTD @ 10,440'

Est. TOC @ 1700

Not to Scale

By: DSC 4/23/26