| Well Name: | Jericho BKJ State Com #2H |
| --- | --- |
| Location: | Sec. 15 T. 25S R. 27E , 660 FNL 330 FEL |
| County: | Eddy, NM |
| Lat/Long: | 32.1356, -104.1708 |
| API #: | 30-015-37500 |
| Spud Date: | RH 2/1/2010; RT 6/18/10 |
| Compl. Date: | 9/20/10 |

Current Wellbore Diagram:

KB: 3190

GL: 3172

17-1/2" Hole

13-3/8" 48# H-40 @ 470'

Cmt w/ 365 sx Class C (circ) to surface

12-1/4" Hole

9-5/8" 36# J-55 to 2232'

Cmt w/ 760 sx Class C (circ) to surface

DV tool @ 5,496'

KOP @ 5710

Pert @ 150'

PLUG: Sqz 64 sxs from 150' to 0'

15 sxs inside, 49 sxs outside

Pilot hole plug from 5380' - 5880'

300 sxs class H

Tag at 5569, slide to 5710, then start directional drill

8-3/4" Pilot Hole to 8497' TVD

Cement Plug @ 6600'-6815'

Perf @ 520' for surf shoe plug

PLUG: Sqz 51 sxs from 520' to 399'

12 sxs in, 39 sxs out. WOC/TAG.

Perf @ 784'

PLUG: Squeeze 64 sxs from 784' to 632'

15 sxs in, 49 sxs out

This will cover top of salt

| Formation Tops |  |
| --- | --- |
| Top of Salt | 734 |
| Bottom of Salt | 2117 |
| Bell Canyon | 2358 |
| Cherry Canyon | 3174 |
| Brushy Canyon | 4250 |
| Bone Spring | 5828 |
| Leonard | 5926 |

Perf @ 2282', attempt to inject

PLUG: If able to inject, sqz 102 sxs class C from 2282-2040

24 sxs in, 78 sxs out. WOC/TAG.

This will cover the int. csg shoe and top of Delaware

PLUG: Spot 25 sxs class C from 3224 - 2971

This will cover the Cherry Canyon.

PLUG: Spot 25 sxs class C from 4300 - 4047

This will cover the Brushy Canyon.

Set CIBP @ 5710 (KOP)

Tag, test casing to 500 psi for 30 min

PLUG: Spot 35 sxs class C on CIBP from 5710 5356. WOC/TAG. This will cover the CIBP and DV tool.

Top Perf 6,384'

Bottom Perf @ 10,434'

TD 10495

Pilot Hole TD @ 8,500'

8-3/4" Lateral Hole

5.5" 17# L80 Prod CSG

to 10,490'. 1380 sx cmt

PBTD @ 10,440'

Not to Scale

By: DSC 4/23/26