Permit Comments

Well: JERICHO BKJ STATE COM #002H

API:30-015-37500

| Comment Date |  |  |
| --- | --- | --- |
| MSANDERS | MUD PROGRAM: 0-400' Fresh Water; 400'-2400' Brine Water; 2400'-8500' Cut Brine; 8500'-11375' Cut Brine (lateral). | 1/5/2010 |
| MSANDERS | BOPE PROGRAM: 3000# psi system nippled up on 13 3/8" and 9 5/8" casing. | 1/5/2010 |
| MSANDERS | MAXIMUM ANTICIPATED BHP: 0-400' 191 PSI; 400'-2400' 1248 PSI; 2400'-8500' 4199 PSI. Depths are TVD. | 1/5/2010 |
| MSANDERS | Pilot hole drilled vertically to 8500'. Well will be plugged back with 400'-500' kick off plug at approx. 6483' and directionally drilled at 12 degrees per 100' with a 8 3/4" hole to 7233' MD (6960' TVD). If hole conditions dictate, 7" casing will be set. | 1/5/2010 |
| MSANDERS | A 6 1/8" hole will then be drilled to 11375' MD (6960' TVD) where 4 1/2" casing will be set and cemented. If 7" casing is not set, then hole size will be reduced to 7 7/8" and drilled to 11375' MD (6960' TVD) where 5 1/2" casing will be set and cemented. | 1/5/2010 |
| MSANDERS | Penetration point of producing zone will be encountered at 660' FNL and 808' FEL, 15-25S-27E. Deepest TVD in the well is 8500' in the pilot hole. Deepest TVD in the lateral will be 6960'. | 1/5/2010 |
| MSANDERS | C-144; Casing Design Plan and Drilling Plan to follow by mail. | 1/5/2010 |