Production

|  | 0 ft to 6,400 ft | Make up Torque ft-lbs | Total ft = 6,400 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. min. | mx. |  |
| 5.5 inches | 17#/ft | P-110 | LT&C | 4620 | 3470 | 5780 |
| Collapse Resistance 7,480 psi | Internal Yield 10,640 psi | Joint Strength 445,000# | Body Yield 546,000# | Drift 4,767 |  |  |

|  | 6,400 ft to 10,200 ft | Make up Torque ft-lbs | Total ft = 3,800 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. min. | mx. |  |
| 5.5 inches | 17 #/ft | L-80 | LT&C | 3410 | 2560 | 4260 |
| Collapse Resistance 6,290 | Internal Yield 7,740 psi | Joint Strength 338,000 # | Body Yield 397,000 # | Drift 4.767 |  |  |

Production hole size will be 8 3/4" from drill out of interm. csg to TD of the pilot hole Hole will then be plugged back and kicked off at approx. 5647' with a 8 3/4" hole and drilled to 6.400' MD(6,125 TVD), hole size will then be reduced to 7 7/8" and drilled to 10,200' MD (6,125' TVD) where 5 1/2" casing will be run and cemented.

Contingency Casing Design

If hole conditions dictate, 7" casing will be set at 6,400' MD (6,125' TVD). A 6 1/8" hole will then be drilled to 10,200' MD (6,125' TVD) where 4 1/2" casing will be set and cemented with one stage up to dv tool. After completion procedures, the 4 1/2" casing will be cut and pulled at 5600'.

2nd Intermediate

|  | 0 ft to 100 ft | Make up Torque ft-lbs | Total ft = 100 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. | min. mx. |  |
| 7 inches | 26#/ft | J-55 | LT&C | 3670 | 2750 | 4590 |
| Collapse Resistance | Internal Yield | Joint Strength | Body Yield | Drift |  |  |
| 4,320 psi | 4,980 psi | 367,000# | 415,000# | 6.151 |  |  |

|  | 100 ft to | 5,900 ft | Make up Torque ft-lbs | Total ft = | 5,800 |
| --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | min. mx. |  |
| 7 inches | 23#/ft | J-55 | LT&G | 3130 2350 3910 |  |
| Collapse Resistance 3,270 | Internal Yield 4,360 psi | Joint Strength 313,000# | Body Yield 366,000# | Drift 6,25 |  |

|  | 5,900 ft to 6,400 ft | Make up Torque ft-lbs | Total ft = 500 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. min. | mx. |  |
| 7 inches | 26 #/ft | J-55 | LT&C | 3670 | 2750 | 4590 |
| Collapse Resistance 4,320 psi | Internal Yield 4,980 psi | Joint Strength 367,000 # | Body Yield 415,000 # | Drift 6,151 |  |  |

Lead w/450sx Lite crete (YLD 2.66 Wt. 9.9) tail w/125sx PVL (YLD 1.41 Wt 13) TOC = 1900'

Production

|  | 0 ft to 10,200 ft | Make up Torque ft-lbs | Total ft = 10,200 |  |  |
| --- | --- | --- | --- | --- | --- |
| O.D. | Weight | Grade | Threads | opt. min. mx. |  |
| 4.5 inches | 11.6 #/ft | HCP-110 | LT&C | 3020 2270 3780 |  |
| Collapse Resistance 8,650 psi | Internal Yield 10,690 psi | Joint Strength 279,000 # | Body Yield Drift 367,000 # |  |  |

DV tool placed at approx. 5600' and cemented with one stage up to dv tool. After completion procedures, the

4 1/2" casing will be cut and pulled at 5600'.

Cemented w/625sx PVL (YLD 1.41 Wt 13) TOC= 5600'