| Submit 1 Copy To Appropriate District Office\nDistrict I\n1625 N French Dr, Hobbs, NM 88240\nDistrict II\n1301 W Grand Ave, Artesia, NM 88210\nDistrict III\n1000 Rio Brazos Rd, Aztec, NM 87410\nDistrict IV\n1220 S St Francis Dr, Santa Fe, NM\n87505 | State of New Mexico\nEnergy, Minerals and Natural Resources | Form C-103\nOctober 13, 2009 |
| --- | --- | --- |
| OIL CONSERVATION DIVISION\n1220 South St. Francis Dr.\nSanta Fe, NM 87505 | WELL API NO.\n30-015-37500 |  |
| 5. Indicate Type of Lease\nSTATE ☒ FEE ☐ |  |  |
| 6. State Oil & Gas Lease No. |  |  |
| SUNDRY NOTICES AND REPORTS ON WELLS\n(DO NOT USE THIS FORM FOR PROPOSALS TO DRILL OR TO DEEPEN OR PLUG BACK TO A DIFFERENT RESERVOIR. USE "APPLICATION FOR PERMIT" (FORM C-101) FOR SUCH PROPOSALS )\n1. Type of Well: Oil Well ☒ Gas Well ☐ Other | 7. Lease Name or Unit Agreement Name\nJericho BKJ State Com |  |
| 8. Well Number\n2H |  |  |
| 2. Name of Operator\nYates Petroleum Corporation\nRECEIVED\nJUL 7 2010 | 9. OGRID Number\n025575 |  |
| 3. Address of Operator\n105 South Fourth Street, Artesia, NM 88210 | 10. Pool name or Wildcat\nUndesignated; Bone Spring |  |
| 4. Well Location\nUnit Letter A : 660 feet from the Section 15 | 30 feet from the East line\nNMPM Eddy County |  |
|  | 11. Elevation (Show whether DR, RKB, RT, GR, etc.)\n3172'GR |  |

12. Check Appropriate Box to Indicate Nature of Notice, Report or Other Data

NOTICE OF INTENTION TO:

PERFORM REMEDIAL WORK ☐ PLUG AND ABANDON ☐

PERFORM REMEDIAL WORLD TEMPORARILY ABANDON PULL OR ALTER CASING DOWNHOLE COMMINGLE

SUBSEQUENT REPORT OF:

□ CHANGE PLANS

□ ALTERING CASING □

□ MULTIPLE COMPL

$ \underline{\text{OTHER}} $:

OTHER: Surface and intermediate casing

13. Describe proposed or completed operations. (Clearly state all pertinent details, and give pertinent dates, including estimated date of starting any proposed work). SEE RULE 19.15.7.14 NMAC. For Multiple Completions: Attach wellbore diagram of proposed completion or recompletion.

6/18/10 – Resumed drilling at 7:30 AM. TD 17-1/2" hole to 470'. Set 13-3/8" 48# H-40 ST&C casing at 470'. Cemented with 165 sx Class “C” + 0.125#/sx D130 + 4% D20 + 2% S1 + 0.2% D46 (yld 1.96, wt 12.9). Tailed in with 200 sx Class “C” + 0.5% S1 (yld 1.34, wt 14.8). Circulated 120 sx to pit. Tested casing to 600 psi for 30 min. WOC 22 hrs 45 min. Reduced hole to 12-1/4" and resumed drilling.

6/21/10 – TD 12-1/4" hole to 2232'. Set 9-5/8" 36# J-55 LT&C casing at 2232'. Cemented with 560 sx 35:65:6 Poz "C" + 0.125#/sx D130 + 2#/sx D142 + 6% D20 + 5% D44 + 0.2% D46 (yld 2.08, wt 12.6). Tailed in with 200 sx Class "C" (yld 1.33, wt 14.8). Circulated 140 sx to pit. Tested casing to 1200 psi for 30 min. WOC 20 hrs 36 min. Reduced hole to 8-3/4" and resumed drilling.

Spud Date:

2/1/10

Rig Release Date:

I hereby certify that the information above is true and complete to the best of my knowledge and belief.

SIGNATURE

TITLE Regulatory Compliance Supervisor DATE July 1, 2010

Type or print name  $ \underline{\text{Tina Huerta}} $ E-mail address:  $ \underline{\text{tinah@yatespetroleum.com}} $ PHONE:  $ \underline{\text{575-748-4168}} $ For State Use Only

APPROVED BY: Donald May TITLE Field Supervisor DATE 7-8-10 Conditions of Approval (if any):