Page 01/02

Tie-in Date: 06/19/2010

Date Completed: 06/28/2010

Survey Horiz. Reference: WELLHEAD

Ref Coordinates: LAT:32.8.7.9146 N LON:104.10.14.5000 W

GRID Reference: NAD83 new mexico east Transverse Mercator

Ref GRID Coor: X: 591679.3420 Y: 413080.2210

YATES PEIHOLEUM

JERICHO BKJ STATE COM #2H - PILOT HOLE

EDDY COUNTY, NEW MEXICO

North Aligned To: GRID NORTH

Survey Calculations by RX5 V6.03A using Minimum Curvature

| Measured Depth (ft) | Incl (deg) | Drift Dir. (deg) | TVD (ft) | Course Length (ft) | Vertical Section (ft) | TOTAL Rectangular Offsets (ft) | Closure Dist (ft) | Dis Dist (deg) | DLS (deg/100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASSUMING VERTICAL DEPTH @ 422' MD. |  |  |  |  |  |  |  |  |  |
| 422.00 | 0.00 | 0.00 | 422.00 | 0.00 | 0.00 | 0.00 | 0.00@ | 0.00 | 0.00 |
| THE FOLLOWING ARE PATHFINDER MWD SURVEYS. |  |  |  |  |  |  |  |  |  |
| 522.00 | 1.23 | 279.05 | 522.00 | 2.00 | 0.02 | 0.00 N | 0.02 W | 0.02@ | 279.05 |
| 706.00 | 2.29 | 283.15 | 705.91 | 184.00 | 5.56 | 1.15 N | 5.55 W | 5.67@ | 281.71 |
| 890.00 | 1.23 | 282.19 | 889.82 | 184.00 | 11.07 | 2.40 N | 11.06 W | 11.32@ | 282.26 |
| 1074.00 | 1.49 | 281.00 | 1073.77 | 184.00 | 15.36 | 3.28 N | 15.34 W | 15.69@ | 282.06 |
| 1257.00 | 2.11 | 280.64 | 1256.68 | 183.00 | 21.01 | 4.35 N | 20.99 W | 21.43@ | 281.72 |
| 1449.00 | 2.55 | 272.64 | 1448.52 | 192.00 | 28.75 | 5.20 N | 28.73 W | 29.19@ | 280.27 |
| 1639.00 | 0.62 | 220.50 | 1638.44 | 190.00 | 33.64 | 4.62 N | 33.62 W | 33.93@ | 277.82 |
| 1830.00 | 1.41 | 119.44 | 1829.42 | 191.00 | 32.26 | 2.67 N | 32.24 W | 32.35@ | 274.74 |
| 2019.00 | 3.34 | 103.73 | 2018.25 | 189.00 | 24.87 | 0.22 N | 24.87 W | 24.87@ | 270.52 |
| 2178.00 | 2.46 | 82.33 | 2177.05 | 159.00 | 16.98 | 0.42 S | 16.99 W | 16.99@ | 268.59 |
| 2284.00 | 2.20 | 75.53 | 2282.96 | 106.00 | 12.76 | 0.39 N | 12.76 W | 12.77@ | 271.76 |
| 2537.00 | 0.88 | 356.49 | 2535.89 | 253.00 | 8.20 | 3.55 N | 8.18 W | 8.91@ | 293.44 |
| 3012.00 | 0.79 | 20.18 | 3010.84 | 475.00 | 7.33 | 10.26 N | 7.27 W | 12.58@ | 324.67 |
| 3488.00 | 0.26 | 47.72 | 3486.82 | 476.00 | 5.42 | 14.07 N | 5.34 W | 15.05@ | 339.21 |
| 3964.00 | 0.18 | 218.12 | 3962.82 | 476.00 | 5.08 | 14.20 N | 5.00 W | 15.06@ | 340.60 |
| 4439.00 | 0.79 | 185.89 | 4437.80 | 475.00 | 5.86 | 10.36 N | 5.80 W | 11.87@ | 330.76 |
| 4915.00 | 0.97 | 81.76 | 4913.76 | 476.00 | 2.19 | 7.67 N | 2.15 W | 7.97@ | 344.36 |
| 5390.00 | 0.97 | 96.81 | 5388.70 | 475.00 | -5.78 | 7.77 N | 5.82 E | 9.71@ | 36.83 |
| 5869.00 | 0.79 | 197.15 | 5867.66 | 479.00 | -8.85 | 4.14 N | 8.87 E | 9.79@ | 65.01 |
| 6336.00 | 0.44 | 85.98 | 6334.65 | 467.00 | -9.71 | 1.19 N | 9.71 E | 9.79@ | 83.04 |
| 6800.00 | 1.32 | 203.05 | 6798.61 | 464.00 | -9.42 | 3.61 S | 9.40 E | 10.07@ | 110.99 |
| 7267.00 | 0.79 | 288.38 | 7265.55 | 467.00 | -4.28 | 7.54 S | 4.24 E | 8.65@ | 150.67 |
| 7735.00 | 5.80 | 31.06 | 7732.76 | 468.00 | -13.31 | 13.75 N | 13.38 E | 19.19@ | 44.22 |